CURTAIN MASTER M6 – A NEW PARTNER IN EFFICIENT LOGISTICS

Wielton’s Curtain Master M6 is a new-generation curtain semi-trailer, developed with durability, transport efficiency and lower operating costs in mind. Thanks to its reduced unladen weight and robust design, the vehicle supports efficient and reliable everyday operation.

The optimised frame and body structure, as well as the standardization of key design solutions, made it possible to reduce the semi-trailer’s unladen weight to 5,640 kg in the basic version. The new aluminium board profile used in the front wall, together with redesigned locks, increases the rigidity of the structure, improves the quality of the connections and transfers loads more effectively. The modified method of connecting the boards to the corner posts makes the structure more resistant to deformation, while also being lighter than in the previous generation of the semi-trailer.

The rear door structure has been thoroughly redesigned, and the use of a hinge with a single pivot point increases its stability and durability in everyday operation. In addition, the fully bolted door structure, without additional rivets, facilitates servicing, while independent hinges make it possible to replace only the damaged component without having to replace the entire set. The plywood floor is additionally bonded, which has reduced the number of screws used and extended its service life. The floor load capacity, confirmed by tests, has increased to 7,600 kg, ensuring high resistance to loads during intensive operation.

The standard version includes the first lift axle, aluminium stanchion boards and door-opening signalling as part of Telematics 2.0 from Aberg Connect. The side raves feature only active Safety-Lock holes, allowing the strap to be fastened along the entire length of the rave and eliminating the need for adapters. In addition, the semi-trailer is equipped with new interior lighting based on LED strips in the corners, as well as robust steel bumpers on the corners and under the handles. Customers can choose from 11 chassis equipment packages in the form of toolboxes or pallet boxes, as well as spare wheel baskets.

The new-generation M6 curtain semi-trailer has been designed to support users in their daily work by ensuring durability, reliability and lower operating costs. By combining an optimised yet solid structure with functional solutions, the Wielton semi-trailer helps increase payload, improve transport efficiency and enhance ease of operation.

The Curtain Master M6 and other vehicles from the brands forming part of the Wielton Group can be viewed at indoor stand E02 in Hall 27 and at the outdoor stands K41 and M67.